ETEC 326: Technical Project (Sec.001)

Final Project Report

Ski-Safety GPS

Professor Mark Thomas

Group 9:

Cong Gu (301201228)

Yubin Jeong (301291041)

Tristan Lai (301284657)

Date: April 16th,2025

Introduction

Our innovative Ski-Safety GPS device combines affordability, reliability, and cutting-edge technology to address critical safety gaps in skiing and winter sports. By integrating GPS tracking, LoRaWAN connectivity, and crash detection into a compact, mount-on design, we deliver a cost-effective solution that enhances skier safety without the burden of subscription fees. Priced under $100, our device stands apart from competitors by targeting the unmet needs of recreational users, expert athletes, and ski resorts, bridging the divide between premium safety gear and accessible technology.

Motivation & Market Need

Why did we create this project?

Skiing and snowboarding are among the most popular winter activities, yet safety remains a persistent concern even for experts. Existing solutions are either vastly more expensive (e.g., avalanche transceivers) or rely on cellular networks with spotty mountain coverage and recurring fees. Our research revealed a clear niche: a low-cost, reliable, and resort-integrated safety device that operates independently of cellular infrastructure.

How does it satisfy the market?

For skiers: Affordable peace of mind with crash detection and emergency alerts.

For resorts: Scalable safety infrastructure with real-time monitoring via LoRaWAN.

For the industry: A subscription-free model that disrupts the status quo.

Solution & Technical Approach

How did we solve the problem?

- Hardware Design:

- STM32 microcontroller: Processes sensor data efficiently.

- RAK LoRaWAN modules: Enable long-range, low-power communication.

- NEO-8M GPS + LIS2DH12 accelerometer: Pinpoints location and detects crashes with high accuracy.

- Network Infrastructure:

- RAK LPS8N LoRaWAN gateway: Provides 5–15 km range in mountainous terrain, operates in extreme temperatures (–40°C to +85°C), and supports hundreds of devices per gateway, eliminating cellular costs.

- Cost Optimization:

- Total BOM cost under $100 by leveraging off-the-shelf components and LoRaWAN’s free frequencies.

Competitive Analysis

We evaluated alternatives like:

- Bluetooth wearables: Limited range (~100m). High power consumption.

- Cellular SOS devices: Expensive subscriptions and unreliable in remote areas.

- Avalanche transceivers: Overkill for resort skiing ($300+).

Our LoRaWAN-GPS hybrid emerged as the only solution balancing range, cost, and reliability.

Development Challenges & Resolutions

1.Battery Life in Cold Weather:

- Optimized ESP32/STM32 firmware for low-power modes and LoRaWAN duty cycling.

2.False Crash Detection:

- Tuned accelerometer thresholds (LIS2DH12/MPU6050) to ignore jumps/vibrations.

3.Gateway Scalability:

- Stress-tested the LP8N gateway to handle peak resort traffic.

4.User Experience:

- Designed a rugged, glove-friendly enclosure with one-button alerts.

System Design & Requirements

What It Does

- Real-time location tracking: GPS updates via LoRaWAN every 30s (adjustable).

- Crash detection: Automatic alerts triggered by impact + motionless timeout.

- Manual SOS: Button press for emergencies.

- Resort integration: Alerts routed to patrol via gateway dashboard.

Key Requirements Met

| Requirement | Solution | Impact |
| --- | --- | --- |
| Under $100 | STM32 + RAK modules | Affordable for mass adoption |
| No subscriptions | LoRaWAN (free frequencies) | Eliminates recurring costs |
| Resort-scale coverage | LP8N gateway (15 km range) | Works in remote slopes |
| All-weather durability | Industrial-grade components | Survives snow, drops, and freezing temps |

Conclusion

Safety and security are fundamental to everyday life, and winter sports are not any exception. The Ski-Safety GPS offers peace of mind to both enthusiasts and beginners by delivering reliable protection and tracking wherever they are on the freezing slopes. By addressing cost, connectivity, and usability gaps, we’ve created a tool that empowers skiers and resorts alike. Future iterations could integrate more complex features from avalanche detection to AI risk analytics, but even in its current form, this project proves that advanced safety can be both accessible and scalable.

Social Innovation Statement

The Ski-Safety GPS tackles a critical societal challenge: making winter sports safety accessible to all. By combining low-cost hardware with LoRaWAN technology, we address the economic and logistical barriers of traditional safety gear, eliminating subscription fees, cellular dependency, and high upfront costs. Our device empowers recreational skiers, resorts, and winter sports communities with reliable crash detection and emergency alerts, reducing response times and preventable injuries or even deaths. By eventually collaborating with ski resorts and outdoor organizations, we’re pioneering a scalable model that prioritizes mass user safety.