LoRanWan Gateway (LSP8N) Setup Manual

Yubin Jeong (301291041)

https://wiki.dragino.com/xwiki/bin/view/Main/User%20Manual%20for%20All%20Gateway%20models/LPS8N%20-%20LoRaWAN%20Gateway%20User%20Manual/

https://www.youtube.com/watch?v=RSdvsrd7-v4&t=455s

1.First is to power up the Gateway and connect your PC’s wifi to draginoxxxx

Enter the passcode:dragino+dragino

2.Then type in http://10.130.1.1/ in your browser

Enter username:root password:dragino

3.Go to internet -> wifi -> wifi survey->choose the network you want->enter the passphrase->enable wifi wan client->see in green: wifi status:ok

4.Go to LoRa->choose the frequency plan->US915->save and apply

5.Go to LoRaWan ->UDP->change the server address to nam1

Find a Ai generated EUI online: (here we used EUI:ca5b966cb0fa1228)save and apply

Now you should have connection every part (physically on the gateway, it should be blue on internet and red on power) on your home page, copy the EUI and registered The Things Stack: https://www.thethingsnetwork.org/

Refresh or click LoRa at http://10.130.1.1 and check the Gateway Status (green means you’re connected)